|  | Simulation of semi_rigid_IMU Date: Thursday, 5 November 2015 Designer: Philip Rowse Study name: SimulationXpress Study Analysis type: Static. Table of Contents Description 2 Assumptions 3 Model Information 3 Material Properties 5 Loads and Fixtures 6 Mesh information 7 Study Results 10 Conclusion 12. |
| --- | --- |
| # Description Design analysis if the foamless Isolator |  |

# Assumptions

# Model Information Model name: semi_rigid_IMU Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Boss-Extrude3; Solid Body; Mass:0.000834655 kg Volume:3.39622e-007 m^3 Density:2457.6 kg/m^3 Weight:0.00817962 N; W:\Git\Pixhawk2\FOAMLESS\semi_rigid_IMU.SLDPRT Nov 05 17:27:51 2015.

# Material Properties Model Reference; Properties; Components. Name:; FR4. Model type:; Linear Elastic Isotropic. Default failure criterion:; Mohr-Coulomb Stress. Yield strength:; Temp dependent. Tensile strength:; 2.55106e+008 N/m^2.; SolidBody 1(Boss-Extrude3)(semi_rigid_IMU).

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-2; Entities:; 4 face(s). Type:; Fixed Geometry.. Load name; Load Image; Load Details. Force-4; Entities:; 1 face(s). Type:; Apply normal force. Value:; 1 N..

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.789292 mm. Tolerance; 0.0394646 mm. Mesh Quality; High. ## Mesh information - Details Total Nodes; 18376. Total Elements; 8667. Maximum Aspect Ratio; 9.5799. % of elements with Aspect Ratio < 3; 84.7. % of elements with Aspect Ratio > 10; 0. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:05. Computer name:; PROFICNC-PC.

# Study Results Name; Type; Min; Max. Stress; VON: von Mises Stress; 1620.49 N/m^2 Node: 18369; 8.20684e+007 N/m^2 Node: 12884. semi_rigid_IMU-SimulationXpress Study-Stress-Stress. Name; Type; Min; Max. Displacement; URES: Resultant Displacement; 0 mm Node: 261; 1.85846 mm Node: 6261. semi_rigid_IMU-SimulationXpress Study-Displacement-Displacement. Name; Type. Deformation; Deformed shape. semi_rigid_IMU-SimulationXpress Study-Displacement-Deformation. Name; Type; Min; Max. Factor of Safety; Max von Mises Stress; 1.2185e-008 Node: 12884; 0.000617098 Node: 18369. semi_rigid_IMU-SimulationXpress Study-Factor of Safety-Factor of Safety.

# Conclusion